CASE STUDY: INSULIN COOLER FOR LIBYA

- Context: Why Libya?

Libya has a high diabetes burden, with roughly 18–19% of adults living with diabetes. Many Middle East and North Africa countries, including Libya, experience inconsistent insulin access due to cost, supply interruptions, and weak health systems. The climate hot: reaches 30–34°C in summer.

Also to consider; weak infastructure and roads

Insulin must be stored between 2–8°C. In Libya this is difficult because:

- Temperature is 35–40°C or higher.

- Power cuts can last many hours.

- Rural patients may travel long distances in desert heat, as proper health centres are only in big citys such as Tripoli, Benghazi and Zawiya.

User groups:

- Urban insulin-dependent patients who live outside of citys.

- Clinics and health posts with weak infrastructure and frequent power cuts.

Typical scenarios:

- A patient in Tripoli needs to keep insulin cold during a 10–12 hour power cut.

- A rural patient travels 5–6 hours in 40°C heat to reach a hospital.

- Clinics require a backup method to maintain the cold chain when generators fail.

- Proposed Solution: Libya-Optimised Insulin Cooler

A portable insulin cooler designed for the Libyan environment. Key features:

- Capacity: 2–4 insulin pens or 4–6 vials.

- Phase Change Material (PCM) that freezes/melts around 5°C to maintain stability.

- High-performance insulation (foam or vacuum panel).

- Multiple recharging methods: standard electricity, 12V car socket, or a small solar panel.

- Cooling duration: 24–72 hours at 40–45°C ambient temperature without power, depending on model.

- Rugged design resistant to dust, sand, and heat.

Use in Libya:

- In cities: acts as a bridge during power cuts.

- In rural areas: PCM can be frozen at clinics (many now have partial solar power) before long journeys.

- For displaced populations: works as an insulin “go-bag” during emergency movement.

- Constraints and Design Requirements in Libya

Environmental constraints:

- Device must operate in sustained 40°C heat and survive peaks near 45–46°C.

- Must resist sand, dust storms, and occasional water splash.

- Should be portable and durable for daily use.

Infrastructure constraints:

- Must function for at least 24 hours without electricity; 48–72 hours is ideal.

- Should use simple components because technical repair capacity is limited.

- Should not depend on specialised refrigerants or parts that are hard to import.